[COLLEGE NAME]

[Department of Computer Science and Engineering]

[University / Affiliation]

MINOR PROJECT SYNOPSIS

on

CYBER CHAUKIDAAR

A Multi-Sensor Perimeter Intrusion Detection and Localisation System

Submitted by

| Name | Roll No. |
| --- | --- |
| Vansh Chawla | [Roll No.] |
| Ishpreet Singh | [Roll No.] |
| Prabhmehar Singh | [Roll No.] |
| Tarandeep Singh | [Roll No.] |

Under the guidance of: Ms. Basanti [Surname], [Designation]

Course / Semester: [B.Tech, Semester ]

Academic Year: 2025–26

# 1. Introduction

Physical security of properties such as farms, warehouses, campuses and remote installations still depends largely on cameras and human guards. Cameras fail in darkness, fog and dense vegetation, need continuous power, and produce large volumes of footage that nobody watches in real time. This project proposes a low-cost, battery-powered sensor grid that passively detects human and vehicle movement near a perimeter, estimates where the intruder is and which way they are moving, and alerts an operator on a live dashboard and on a phone.

Each sentry node combines a 24 GHz human-presence radar with two accelerometers coupled to the ground through steel spikes, which pick up footstep vibrations through the soil. A lightweight machine-learning model on the node distinguishes footsteps from vehicles and wind, and a central server running on a Raspberry Pi fuses all evidence, localises the target on a site map and raises alarms. The platform also includes an OSINT breach checker that lets the operator verify whether staff credentials have appeared in known data leaks, so that the physical and digital sides of site security are covered in one tool.

# 2. Problem Statement

Existing perimeter solutions are either expensive (thermal cameras, fibre-optic fences), power-hungry, or prone to false alarms from animals and weather. There is a need for an affordable system that:

- detects humans and vehicles at 5–8 m without line of sight or lighting,

- gives a rough position and direction of movement rather than only a binary alarm,

- runs for weeks on a single battery,

- scales to many nodes on one dashboard, and

- explains why an alarm was raised so that operators can trust it.

# 3. Objectives

- Design a battery-efficient sentry node (ESP32-S3, HLK-LD2420 radar, two ADXL345 accelerometers on ground spikes) with sleep/wake power management.

- Implement on-device signal processing: band-pass filtering, STA/LTA event picking, spectral feature extraction and a random-forest classifier for footsteps, vehicles and environmental noise.

- Estimate the intruder's bearing from the two accelerometers (time difference of arrival and amplitude ratio) and range from the radar, producing an estimated position on a site map.

- Build a central server that fuses radar, seismic and machine-learning evidence into an intrusion probability, tracks targets across nodes and manages alerts.

- Develop a real-time dashboard showing every node's sensor values, radar scope, seismic waveforms, telemetry (battery, voltage, signal strength) and detections, with arming modes, zones and notifications.

- Provide tamper detection, loitering detection and local deterrence (LED ring and buzzer) using the same hardware.

# 4. Proposed System and Methodology

## 4.1 Sentry node

The ESP32-S3 samples both ADXL345 accelerometers at 100 Hz over SPI. The vibration magnitude is band-pass filtered (1.5–40 Hz) and fed to a short-term/long-term average (STA/LTA) trigger. Every second a 128-sample window is converted into ten features (RMS, peak, variance, dominant frequency, spectral centroid, inter-peak interval, crest factor and others) and classified by a random forest exported as compact C tables. Cross-correlation between the two probes gives the arrival-time lag which, together with the amplitude ratio, yields the direction of the source. The LD2420 radar reports presence and distance. In ECO mode the node light-sleeps between sensor reads and sends only one heartbeat per minute; any trigger switches it to ACTIVE mode with one packet per second over Wi-Fi (BLE and LoRa transports are also implemented). A WS2812 LED ring points toward the detected target and, with a buzzer, acts as a local deterrent.

## 4.2 Central server

A Node.js server on a Raspberry Pi normalises incoming packets, re-classifies them for verification and fuses the evidence with a logistic-regression model into an intrusion probability with an itemised evidence list. Rule guards prevent a single sensor from raising an alarm on its own. The localiser converts range and bearing into site coordinates with an uncertainty ellipse; detections from neighbouring nodes are merged into tracks with speed, heading, a five-second prediction and, when two radars see the same target, a triangulated fix. Alerts support debouncing, zones (restricted / watch / allowed), arming schedules, tamper and loiter kinds, incident replay, and Telegram, webhook and GPIO siren notifications.

## 4.3 Dashboard

A React web dashboard provides a site map with nodes, radar beams, probe energy rings, estimated positions and tracks; a radar scope; a dual-trace seismic oscilloscope; a telemetry column; fusion explainability; an incident log with replay and CSV export; and a setup page for node placement and zones. A physically consistent simulator allows additional nodes to be demonstrated before hardware is available.

## 4.4 Machine learning

The models are trained in pure Python on synthetic footstep, vehicle and environmental waveforms and can be retrained on real windows recorded through the dashboard. The random-forest classifier reached 97.8% accuracy on held-out data and is exported for both the server (JSON) and the microcontroller (C header).

# 5. System Architecture

| Layer | Components | Function |
| --- | --- | --- |
| Sensing | LD2420 radar, 2 × ADXL345 on ground spikes | Presence and range; footstep / vehicle vibration |
| Edge processing | ESP32-S3 firmware (C++) | Filtering, STA/LTA, features, random forest, bearing, power management, uplink |
| Server | Raspberry Pi, Node.js | Fusion, localisation, tracking, alerts, zones, notifications, OSINT breach checker |
| Presentation | React dashboard | Live map, scopes, telemetry, incidents, setup |

# 6. Hardware and Software Requirements

| Hardware | Software |
| --- | --- |
| ESP32-S3 development board | PlatformIO with Arduino framework (C++) |
| 2 × ADXL345 accelerometers with steel ground spikes | Node.js 20, Express, WebSocket |
| HLK-LD2420 24 GHz human-presence radar | React, Vite, Tailwind CSS |
| WS2812B LED ring, buzzer | Python 3 (model training) |
| 18650 Li-ion cell, charger, voltage regulator | Raspberry Pi OS, systemd |
| Raspberry Pi (server and gateway) | Git / GitHub |

# 7. Expected Outcomes

- Detection of walking humans at 5–8 m and vehicles at greater range, with radar and seismic confirmation.

- Position estimate within about one metre when the radar has range, and correct left/right bearing from the probe pair.

- Battery life of the order of weeks in ECO mode.

- A working multi-node dashboard with explainable alerts, replay and phone notifications.

# 8. Applications

Farm and orchard protection, warehouse and construction-site security, campus and residential perimeters, wildlife-corridor monitoring, and border or forest-post early warning where cameras are impractical.

# 9. Future Scope

Multi-node time synchronisation for full triangulation from seismic data alone, on-device model retraining, solar charging, camera hand-off on alarm, and a LoRa mesh for large sites.

# 10. Conclusion

Cyber Chaukidaar demonstrates that inexpensive sensors, careful signal processing and lightweight machine learning can provide reliable, explainable perimeter awareness without cameras or mains power, while a single dashboard covers both the physical perimeter and the digital exposure of the people guarding it.

# 11. References

- Analog Devices, "ADXL345: 3-Axis, ±2 g/±4 g/±8 g/±16 g Digital Accelerometer", Datasheet.

- Hi-Link Electronic, "HLK-LD2420 24 GHz Human Presence Radar Module", User Manual.

- Espressif Systems, "ESP32-S3 Technical Reference Manual".

- R. V. Allen, "Automatic earthquake recognition and timing from single traces", Bulletin of the Seismological Society of America, vol. 68, no. 5, 1978.

- L. Breiman, "Random Forests", Machine Learning, vol. 45, pp. 5–32, 2001.

Signature of Students	Signature of Mentor

Date: ____________	Ms. Basanti [Surname]